Question Bank (I-Scheme)

Name of course: Basic Electronics                                                  Unit Test: I

Subject code: 22225 (BEC)                Semester: II                         Program: CM/IF

CHAPTER N0: 1

- Define Active and Passive components.

- Compare Active and Passive Components.

- Draw the symbols of following components:

- Resistor     b) Capacitor     c) Inductor

- Write down color code for following resistor  values:

- 270Ω;  ±10%

- 5.6KΩ;  ±5%

- 100Ω; ±10%

- 47KΩ; ±5%

- State the functions of Inductor.

- State the functions of Capacitor.

- List the specifications of following components:

- Resistor     b) Capacitor     c) Inductor

- Define the terms:

- Amplitude     b) Frequency    c) Phase    d) Wavelength

- Sketch ideal and practical current source and voltage source.

- Define Integrated circuits. State different scales of Integration.

- Compare Analog IC & Digital IC.

- Draw the waveforms of sinusoidal, Square & Triangular analog signals & Label it properly.

CHAPTER: 2 (Diodes & their Applications)

2 marks questions

- Define :   a) P-N  Junction Diode     b) Zener Diode

- Define :   a) Rectifier     b) Filter

- Draw the Symbols of  a) P-N  Junction Diode     b) Zener Diode        c) LED

- Draw constructional diagram of LED.

- List the applications of zener diode & LED.

- State two advantages of full wave rectifier.

- State the need of Filters.

4 marks Questions

- Explain working of P-N Junction diode, when it is connected in forward bias.

- Explain working principle of zener diode with V-I characteristics.

- Explain working principle of inductor filter with its neat circuit diagram.

- Explain working principle of capacitor filter with its neat circuit diagram.

- Explain working principle of π filter with its neat circuit diagram.

- Compare L, C, & π filter with respect to given features:

- Components used               d) place of filter

- Output Dc voltage               e) ripple factor

- Load regulation                     f) Applications

- Explain working principle of Half Wave Rectifier with a neat circuit diagram.

- Explain working principle of Bridge Rectifier with a neat circuit diagram.

- Compare Half Wave & Full Wave Rectifiers with respect to given parameters:

- No. of Diodes

- PIV

- Ripple Factor

- Rectification Efficiency

- Output Waveforms

- Define the following terms:

a) PIV      b) Ripple Factor   c) Rectification Efficiency d) Ripple

11) Sketch the block diagram of regulated power supply & explain its working principle.

12)  Explain Zener diode as a Voltage Regulator with neat circuit diagram.

Chapter: 3 Bipolar Junction Transistors

2 mark questions:

- Define Transistor & Draw symbol of NPN Transistor.

- Draw the construction of NPN Transistor.

- Define:  a) Current gain( α)    b) Current gain( β)

- Draw the output characteristics of Transistor in CE mode.

- Define DC load line & Operating point.

- State need of biasing.

- State the advantages of voltage divider biasing over base bias.

- State the applications of RC coupled CE amplifier.

4 mark Questions

- Explain working principle of NPN Transistor.

- Derive the relationship between α & β of transistor.

- Draw & explain voltage divider biasing.

- Compare CB & CE configuration of transistor.

- If α of a transistor is 0.9, calculate β.

- If β  of a transistor is 98, Ic=  4.5mA, then  calculate base current.

- State the significance of operating point in Transistor biasing.

- Explain how transistor can be used as switch.

- Draw & explain single stage CE amplifier.

- In a voltage divider biasing ckt, Vcc=10V, R1=10KΩ, R2=5Ω, & RE= 100Ω, RC= 1 KΩ, calculate Ic & Vce.

(Consider β=100, Vbe= 0.7v)

Chapter: 4 (FIELD EFFECT TRANSISTOR)

2 mark questions:

- State different types of FET.

- Draw the symbol of N-channel & P-channel JFETs.

- Draw the symbol of Depletion type & Enhancement type MOSFET.

- State the application of JFET.

- State the application of MOSFET.

4 mark questions:

- Compare FET & BJT.

- Draw & Explain N-channel JFET construction.

- Draw & Explain P-channel JFET construction.

- Explain Working Principle of N-channel JFET.

- Draw & Explain Transfer characteristics of JFET.

- Define following terms:

- Dynamic Drain Resistance

- Amplification Factor

- Transconductance

- Pinched off voltage

- Explain Working Principle of Depletion type MOSFET (n-channel).

- Explain Working Principle of Enhancement type MOSFET (n-channel).

Question bank 2

- Marks questions

- Draw the construction of NPN Transistor.

- Define:  a) Current gain( α)    b) Current gain( β)

- Draw the output characteristics of Transistor in CE mode.

- Define DC load line & Operating point.

- State need of biasing.

- State different types of FET.

- Draw the symbol of N-channel & P-channel JFETs.

- Draw the symbol of Depletion type & Enhancement type MOSFET.

- State the application of JFET

- Define Transducer

- Define Active & Passive Transducer

- Marks Questions

- Derive the relationship between α & β of transistor.

- Draw & explain voltage divider biasing

- If β  of a transistor is 98, Ic=  4.5mA, then  calculate base current, emitter current & α

- Draw & Explain N-channel JFET construction.

- Explain Working Principle of N-channel JFET.

- Explain selection Criteria for Transducer.

- Explain Resistive Transducer With an example.

- Explain Inductive Transducer With an example.