BHARATI VIDYAPEETH INSTITUTE OF TECHNOLOGY

Question Bank (I-Scheme)

Name of subject: Elements of Electrical Engg.               Unit Test :I

Subject code: 22215                                                           Course : EJ/CM/IF

Semester: II

CHAPTER-1(Magnetic Circuits)

(2 Marks)

- Define Reluctance .What is its unit?

- Define i) Magnetic flux, ii) Magnetic Flux density.

- State the Lenz’s law. Give expression for the induced voltage.

- State Faraday’s law of Electromagnetic Induction.

- State Fleming’s right hand rule.

(4 Marks)

- Define Self Induced EMF and Mutually Induced EMF with neat sketch.

- Compare Magnetic Circuit and Electric Circuit.

- Write the symbol and unit of following:

- Magnetic flux density

- Magnetic field strength

- Reluctance

- Self inductance

- Explain B-H Curve and Draw with all parameters.

- Find relectance, flux, M.M.F required and exciting current for an iron ring with 200 turns having diameter of 15 cm and 10 cm2 cross sectional area if flux density 1 wb/m2 and permeability of 500.

CHAPTER-2(AC Fundamentals)

(2 Marks)

- Write any two advantages of AC over DC.

- Define Peak Factor and Form Factor.

- Define leading and lagging phase difference.

- An Alternating current is given by i=28.28sin(2∏ 50t) Find R.M.S value and Average value.

- Draw the Impedance Triangle of series R-C circuit.

(4 Marks)

- Define Active Power, Reactive Power, Apparent Power and Power factor.

- For RC circuit:

- Draw the circuit diagram

- Write voltage and current equations

- Draw phasor diagram

- Draw power triangle

- A resistance of 5Ω and capacitive reactance of 10Ω are connected in series if the current through circuit is 3A. find i)Total voltage ii) power factor and its nature. Assume f = 50Hz.

CHAPTER-3(Polyphase AC Circuits)

(2 Marks)

- Define Phase Sequence.

- State the meaning of 3 phase Balanced and Unbalanced Load.

- State any four advantages of Poly phase Circuit over Single Phase circuit.

- Draw 3- phase voltage waveform of a.c. supply with respect to time.

(4 Marks)

- Draw delta connected load. State relation between:

i) Line voltage and phase voltage. ii) Line current and phase current.

- A balanced 3-phase star connected load consist of three resistances each of four ohms connected to 400V, 3 phase 50 Hz supply, find (i) Phase voltage (ii) Phase current (iii) Line current (iv) Power consumed.

- Compare star and delta connected system (4 points)

************************************************