Question Bank (I scheme)

Name of Subject: Embedded Systems (ESY)                                           	     Unit Test: II

Subject Code: 22532                                                                                           Course:  EJ5I                                                                                                   Semester:  V

Chapter 4 : Interfacing Input and Output devices (18 marks)

2 Marks

- Draw interfacing diagram of 16X2 LCD Display with 89C51and state the functions of:                                         i) RS          ii) EN                 iii) R/W     iv) VEE

- Draw interfacing diagram to interface 89C51 microcontroller with four 7- Segment LED display.

- Draw labeled interfacing diagram to interface 4x4 keyboard matrix with microcontroller 89C51

4 Marks

- Draw labeled interfacing diagram to interface LED to P2.0 of 89C51. Write89C51 ‘C’ program to turn on and off LED after some delay.

- Draw labeled interfacing diagram to interface 4x4 keyboard matrix with microcontroller and write a C program to read it.

- Draw the labeled diagram of interfacing DAC with 89C51 microcontroller and Write a program in ‘C’ language for generating triangular waveform using DAC 0808.

- Draw labeled interfacing diagram of ADC 0808 with 89C51 microcontroller. Write 89C51 ‘C’ Program to read the analog data.

- Write 89C51 ‘C’ Program to rotate stepper motor 900 in clock wise direction. Motor has step angle of 1.80. Use the stepper motor in full step sequence.

- Draw labeled interfacing diagram write a program to display welcome to LCD

- Draw interfacing diagram to interface 89C51 microcontroller with four 7- Segment LED display.

- Draw labeled interfacing diagram to interface DC motor with 89C51microcontroller and write a program in ‘C’ to rotate anticlockwise by 1800.

Chapter 5: Real Time Operating Systems (12 marks)

2 Marks

- Differentiate RTOS with desktop operating system (Any four points).

- Describe any 4 specifications of RTOS. Give any 4 examples of RTOS.

4 Mark

- Explain inter process communication in brief. State various inter process communication methods.

- Describe round robin scheduling algorithm with suitable diagram.

- List scheduling algorithms of RTOS. Describe concept of Pre-emptive multitasking scheduling algorithm of RTOS with suitable diagram.

- State the methods of Task synchronization. Describe Semaphore with suitable example.

- Explain the concept of deadlock with suitable example. Describe any three methods each to detect and prevent deadlock.

- With the help of neat diagram describe binary semaphore.

- Explain the following a) watchdog timer    b) multitasking

- With the help of neat diagram describe mutex semaphore.

- Describe any four characteristics of RTOS.