Question Bank (I scheme)

Name of Course: Heat Transfer Operation                            Subject code: 22510

Semester : Fifth                                                                           Programme: Chemical

Unit test I

Unit 1- Conduction (12marks)

2 marks question

- Define thermal conductivity and write its SI unit.

- Define conduction with example

- Define steady state and unsteady state conduction.

- Define insulating materials and give two examples

- Give the effect of temperature on thermal conductivity.

4marks question

- Explain optimum thickness of insulation

- Explain Fourier’s law with mathematical expression.

- Derive an expression to calculate rate of heat flow through a cylinder.

- A wall is made of brick(k= 1.0 W/mK) 230 mm thick. It is lined on the inner face with plaster  (k= 0.4 W/mK) and of thickness 10 mm. If a temperature difference of 30 K is maintained between the two faces, what is the heat flow per unit area of wall?.

Unit 2- Convection (20 marks)

- marks question

- Define convection with example.

- Differentiate natural and forced convection.

- Define film wise condensation and drop wise condensation.

- Give the unit of U in SI and MKS system.

- Heat transfer coefficients are low in film wise condensation. Why?

- Give the expression for LMTD for co current and counter current flow.

- marks question

- Write down the following equations and explain the terms: i) Sider Tate equation ii) Dittus Bolter equation.

- Derive Q= U.A.ΔTLM

- Calculate U from following data:

hi = 5800 W/m2K. ho = 1750 W/m2K

do = 30 mm di = 20 mm

k= 46.52 W/mK

- Differentiate between co current and counter current flow .

- Thermic fluid flowing at a rate of 5000 kg/h is to be cooled from 423 K to 363 K by circulating water at a rate of 15000 kg/h. If the water is available at 303K, find the outlet temperature of water. Cp for thermic fluid = 2.72 kJ/kg K.

- A hot fluid enters a double pipe heat exchanger at a temperature of 423 K and is to be cooled to 367 K by a cold fluid entering at 311 K and heated to 339 K. Shall they be directed in parallel or co current flow?

Unit 3- Radiation (8 marks)

- marks question

- Define radiation with example.

- Define black body and grey body.

- Define absorptivity, reflectivity and transmissivity.

4marks question

- Explain Kirchhoff’s law

- Explain any two laws of black body radiation

- Calculate the heat loss by radiation per unit area from an unlagged horizontal steam pipe 50 mm od at 377 K to air at 283K. Emissivity (ε)= 0.9