- A solution boils at a temperature of 380 K and boiling point of water at a pressure in the vapour space is 373 K. Temperature of condensing steam is 399 K. What is the boiling point elevation of the solution and driving force for heat transfer?

- An evaporator is operating at atmospheric pressure. It is desired to concentrate the feed from 5% solute to 20% solute (by weight) at a rate of 5000kg/hr. Dry saturated steam at a pressure corresponding to saturation temperature of 399K is used. The feed is at 298K and boiling point elevation is 5K. Calculate economy.

Latent heat of condensation of steam = 2185 KJ/kg

Latent heat of vaporization of water = 2257 KJ/kg

Specific heat of feed = 4.187 KJ/kg K